# Device Driver_양영식

NXSOL-OJT-2024

Exported on 2024-02-23 13:40:14

Table of Contents

1	INDEX	3

2	0. UPDATE HISTORY	4

3	1. Introduction	5

4	2. Implemented features	6

4.1	2.1 Feature list	6

4.2	2.2 Command manual	6

4.3	2.3 Flow diagram	6

5	3. Target GPIO pins	7

6	4. Source	8

6.1	4.1 Source code	8

6.2	4.2 Add the source code	11

6.3	4.3 Existing APIs used in the source code	11

7	5. Test	12

7.1	5.1 Test scenario	12

7.2	5.2  Test code	13

7.3	5.3 Run test	17

7.4	5.4 Test result	17

8	Reference	20

# INDEX

- INDEX

- 0. UPDATE HISTORY

- 1. Introduction

- 2. Implemented features

  - 2.1 Feature list

  - 2.2 Command manual

  - 2.3 Flow diagram

- 3. Target GPIO pins

- 4. Source

  - 4.1 Source code

  - 4.2 Add the source code

  - 4.3 Existing APIs used in the source code

- 5. Test

  - 5.1 Test scenario

  - 5.2 Test code

  - 5.3 Run test

  - 5.4 Test result

- Reference

# 0. UPDATE HISTORY

| - EVENT | - Date | - Description | - Author |
| --- | --- | --- | --- |
| - create | - 2024.2.20 | - create | - NXKR_YoungSikYang |
| - modify | - 2024.2.21 | - modified the contents according to the template | - NXKR_YoungSikYang |
| - modify | - 2024.2.21 | - corrected wrong indentation - added source code - added gpio-uclass.c reference | - NXKR_YoungSikYang |
| - modify | - 2024.2.22 | - modified implemented features - deleted memory location - modified test scenario | - NXKR_YoungSikYang |
| - modify | - 2024.2.23 | - Modified command manual so that it can be clearer - Corrected a typo in flow diagram - Added more explanation to each section | - NXKR_YoungSikYang |

# 1. Introduction

- Final assignment of u-boot training.

- Board: BTC08(s5p6818)

Utilizing the GPIO device driver implemented in gpio-uclass.c, I have developed features as specified in the assignment requirements and tested them using u-boot commands.

# 2. Implemented features

The features are related to controlling or querying GPIOs.

## 2.1 Feature list

The features can be classified into 3 categories

- GPIO input/output (GPIOA, GPIOB, GPIOC, GPIOD, GPIOE)

- LED control (AP_LED0, AP_LED1)

- Button detection (AP_KEY0)

## 2.2 Command manual

| - Command | - Action |
| --- | --- |
| - nx_gpio led <on¦off> | - Turn the LEDs on or off |
| - nx_gpio <set¦clear¦toggle¦get> <num> | - set¦clear¦toggle¦get <num> GPIO |
| - nx_gpio btn | - Wait for button presss and toggle the LEDs |

## 2.3 Flow diagram

This diagram visualizes the behavior of each command.

# 3. Target GPIO pins

This picture is from Page 6 of tsb1101_control_v100_rel_20181126.pdf

# 4. Source

## 4.1 Source code

#include <common.h> #include <command.h> #include <stdarg.h> #include <errno.h> #include <dm.h> #include <asm/gpio.h> #define BTN 3 #define LED0 4 #define LED1 5 #define ON 0 #define OFF 1 static int gpios_toggle(int gpio, ...) { va_list args; va_start(args, gpio); int ret; while (gpio != -1) { if (gpio_request(gpio, "cmd_nx_gpio")) { printf("Failed to request gpio %i\n", gpio); return -1; } ret = gpio_get_value(gpio); if (ret < 0) { printf("Failed to get gpio %i value\n", gpio); gpio_free(gpio); return ret; } ret = gpio_set_value(gpio, !ret); if (ret < 0) { printf("Failed to toggle gpio %i value\n", gpio); gpio_free(gpio); return ret; } printf("gpio %i value is %d\n", gpio, !ret); gpio_free(gpio); gpio = va_arg(args, int); } va_end(args); return ret; } static int gpios_set_value(int gpio_value, int gpio, ...) { va_list args; va_start(args, gpio); int ret; while (gpio != -1) { if (gpio_request(gpio, "cmd_nx_gpio")) { printf("Failed to request gpio %i\n", gpio); return -1; } ret = gpio_set_value(gpio, gpio_value); if (ret < 0) { printf("Failed to set gpio %i value\n", gpio); gpio_free(gpio); return ret; } printf("gpio %i value is %d\n", gpio, gpio_value); gpio_free(gpio); gpio = va_arg(args, int); } va_end(args); return ret; } int do_nx_gpio(cmd_tbl_t *cmdtp, int flag, int argc, char * const argv[]) { int gpio_value, gpio, ret; /* Not enough args */ if (argc < 2) { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } if (strcmp(argv[1], "led") == 0) { if (argc < 3) { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } if (strcmp(argv[2], "on") == 0) { if (gpios_set_value(ON, LED0, LED1, -1)) return CMD_RET_FAILURE; } else if (strcmp(argv[2], "off") == 0) { if (gpios_set_value(OFF, LED0, LED1, -1)) return CMD_RET_FAILURE; } else { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } } else if (strcmp(argv[1], "gpio") == 0) { if (argc < 4) { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } ret = gpio_lookup_name(argv[3], NULL, NULL, &gpio); if (gpio < 0) { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } if (strcmp(argv[2], "get") == 0) { if (gpio_request(gpio, "cmd_nx_gpio")) { printf("Failed to request gpio %i\n", gpio); return CMD_RET_FAILURE; } gpio_value = gpio_get_value(gpio); if (gpio_value < 0) { printf("Failed to get gpio %i value\n", gpio); gpio_free(gpio); return CMD_RET_FAILURE; } printf("gpio %i value is %d\n", gpio, gpio_value); gpio_free(gpio); } else if (strcmp(argv[2], "set") == 0) { if (gpios_set_value(1, gpio, -1)) return CMD_RET_FAILURE; } else if (strcmp(argv[2], "clear") == 0) { if (gpios_set_value(0, gpio, -1)) return CMD_RET_FAILURE; } else if (strcmp(argv[2], "toggle") == 0) { if (gpios_toggle(gpio, -1)) return CMD_RET_FAILURE; } else { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } } else if (strcmp(argv[1], "btn") == 0) { printf("Waiting for button press...\n"); if (gpio_request(BTN, "cmd_nx_gpio")) { printf("Failed to request gpio %i\n", gpio); return CMD_RET_FAILURE; } /* Polling */ while (1) { gpio_value = gpio_get_value(BTN); if (gpio_value < 0) { printf("Failed to get gpio %i value\n", 3); gpio_free(BTN); return CMD_RET_FAILURE; } if (gpio_value == 0) { gpio_free(BTN); break; } } /* Toggle LEDs */ if (gpios_toggle(LED0, LED1, -1)) return CMD_RET_FAILURE; } else { printf("Usage: %s\n", cmdtp->usage); return CMD_RET_FAILURE; } return CMD_RET_SUCCESS; } U_BOOT_CMD(nx_gpio, 4, 0, do_nx_gpio, "Query and control gpio pins", "\n- led on¦off: Turn the LEDs on¦off\n" "- gpio set¦clear¦toggle¦get <num>: Set¦Clear¦Toggle¦Get <num> gpio\n" "- btn: Wait for button press and toggle 2 LEDs\n" );

Code Block 1 common/cmd_nx_gpio.c

## 4.2 Add the source code

Configure so that the new source code can be compiled and included in u-boot through the build process.

common/Makefile

This adds the source to the compile list.

common/kconfig

This includes the source in u-boot.

And then build

## 4.3 Existing APIs used in the source code

The source code above takes advantage of this API source

https://github.com/NexellCorp/u-boot-2016.01/blob/artik/drivers/gpio/gpio-uclass.c

# 5. Test

Each command is tested by visually checking the color of the LEDs and verifying the input of the target GPIO through the GPIO driver.

## 5.1 Test scenario

| - Case | - Command | - Expectation | - Goal |
| --- | --- | --- | --- |
| - get GPIOA3 | - nx_gpio gpio get 3 | - GPIOA3 is high with button released → button press -> GPIOA3 is low | - GPIO input works |
| - led off | - nx_gpio led off | - LEDs are on → LEDs are turned off | - LED on/off works |
| - led on | - nx_gpio led on | - LEDs are off → LEDs are turned on | - LED on/off works |
| - led abc | - nx_gpio led abc | - Command does not work (invalid command) | - Invalid command does not work |
| - set GPIOA4,5 | nx_gpio gpio set 4 - nx_gpio gpio set 5 | - GPIOA4,5 are 0 → GPIOA4,5 are set to 1(LEDs are turned off) | - GPIO output works |
| - clear GPIOA4,5 | nx_gpio gpio clear 4 - nx_gpio gpio clear 5 | - GPIOA4,5 are 1 → GPIOA4,5 are cleared to 0(LEDs are turned on) | - GPIO output works |
| - toggle GPIOA4,5 | nx_gpio gpio toggle 4 - nx_gpio gpio toggle 5 | - GPIOA4,5 are 0 → GPIOA4,5 are toggled to 1(LEDs are turned off) | - GPIO output works |
| - toggle GPIOA4,5 on button press | - nx_gpio btn | - GPIOA4,5 are 1 → button press -> GPIOA4,5 are toggled to 0(LEDs are turned on) | - GPIO output works |

## 5.2  Test code

#include <common.h> #include <command.h> #include <errno.h> #include <dm.h> #include <asm/gpio.h> static void gpio_get(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: gpio get\n"); *total_tests += 1; /* Given: button connected to GPIO 3 is released */ /* When: check the value of GPIO 3 */ gpio_request(3, "cmd_nx_gpio"); char gpio_value = gpio_get_value(3); /* Then: GPIO 3 is high */ if (gpio_value == 1) printf("GPIO 3 is high.(released)\n"); else { printf("Test failed: GPIO 3 is not high.\n"); return; } printf("Waiting for button press...\n"); /* Given: button connected to GPIO 3 is released */ /* When: button is pressed */ while (1) { gpio_value = gpio_get_value(3); if (gpio_value == 0) break; } /* Then: GPIO 3 is low */ if (gpio_get_value(3) == 0) { printf("Test passed: GPIO 3 is low.(pressed)\n"); ++*passed_tests; } else printf("Test failed: GPIO 3 is not low.\n"); gpio_free(3); } static void led_off(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: led off\n"); *total_tests += 1; /* Given: LEDs are on */ /* When: turn off the LEDs */ run_command("nx_gpio led off", 0); /* Then: GPIO 4 and 5 are high */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio_get_value(4) == 1 && gpio_get_value(5) == 1) { printf("Test passed: LEDs off.\n"); ++*passed_tests; } else printf("Test failed: LEDs are not off.\n"); gpio_free(4); gpio_free(5); } static void led_on(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: led on\n"); *total_tests += 1; /* Given: LEDs are off */ /* When: turn on the LEDs */ run_command("nx_gpio led on", 0); /* Then: GPIO 4 and 5 are low */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio_get_value(4) == 0 && gpio_get_value(5) == 0) { printf("Test passed: LEDs on.\n"); ++*passed_tests; } else printf("Test failed: LEDs are not on.\n"); gpio_free(4); gpio_free(5); } static void led_abc(unsigned int *total_tests, unsigned int *passed_tests) { int ret; printf("Test: led abc\n"); *total_tests += 1; /* Given: LEDs are off */ /* When: turn on the LEDs */ ret = run_command("nx_gpio led abc", 0); /* Then: Invalid command */ if (ret == 1) { printf("Test passed: Invalid command.\n"); ++*passed_tests; } else if (ret == 0) printf("Test failed: Command not invalid.\n"); } static void gpio_set(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: gpio set\n"); *total_tests += 1; /* Given: LEDs are on */ /* When: setting GPIO 4 and 5 */ run_command("nx_gpio gpio set 4", 0); run_command("nx_gpio gpio set 5", 0); /* Then: LEDs are off */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio_get_value(4) == 1 && gpio_get_value(5) == 1) { printf("Test passed: GPIO 4 and 5 set.\n"); ++*passed_tests; } else printf("Test failed: GPIO 4 and 5 are not set.\n"); gpio_free(4); gpio_free(5); } static void gpio_clear(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: gpio clear\n"); *total_tests += 1; /* Given: LEDs are off */ /* When: clear GPIO 4 and 5 */ run_command("nx_gpio gpio clear 4", 0); run_command("nx_gpio gpio clear 5", 0); /* Then: LEDs are on */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio_get_value(4) == 0 && gpio_get_value(5) == 0) { printf("Test passed: GPIO 4 and 5 cleared.\n"); ++*passed_tests; } else printf("Test failed: GPIO 4 and 5 are not cleared.\n"); gpio_free(4); gpio_free(5); } static void gpio_toggle(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: gpio toggle\n"); *total_tests += 1; /* Given: initial state of GPIO 4 and 5 */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); char gpio4_value = gpio_get_value(4); char gpio5_value = gpio_get_value(5); gpio_free(4); gpio_free(5); /* When: toggle GPIO 4 and 5 */ run_command("nx_gpio gpio toggle 4", 0); run_command("nx_gpio gpio toggle 5", 0); gpio_free(4); gpio_free(5); /* Then: GPIO 4 and 5 are toggled */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio4_value != gpio_get_value(4) && gpio5_value != gpio_get_value(5)) { printf("Test passed: GPIO 4 and 5 toggled.\n"); ++*passed_tests; } else printf("Test failed: GPIO 4 and 5 are not toggled.\n"); gpio_free(4); gpio_free(5); } static void btn(unsigned int *total_tests, unsigned int *passed_tests) { printf("Test: btn\n"); *total_tests += 1; /* Given: initial state of GPIO 4 and 5 / the button connected to GPIO 3 is released */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); char gpio4_value = gpio_get_value(4); char gpio5_value = gpio_get_value(5); gpio_free(4); gpio_free(5); /* When: wait for button to be pressed */ run_command("nx_gpio btn", 0); /* Then: LEDs are toggled */ gpio_request(4, "cmd_nx_gpio"); gpio_request(5, "cmd_nx_gpio"); if (gpio4_value != gpio_get_value(4) && gpio5_value != gpio_get_value(5)) { printf("Button pressed and LEDs toggled.\n"); ++*passed_tests; } else printf("Button press not recognized or LEDs not toggled.\n"); gpio_free(4); gpio_free(5); } static void run_test(void (*test)(unsigned int *, unsigned int *), unsigned int *total_tests, unsigned int *passed_tests) { test(total_tests, passed_tests); printf("Press any key for the next test...\n--------------\n"); while (!tstc()) ; getc(); } int do_test_nx_gpio(void) { unsigned int total_tests = 0; unsigned int passed_tests = 0; run_test(gpio_get, &total_tests, &passed_tests); if (passed_tests == 0) /* Other tests depend on this one */ return; run_test(led_off, &total_tests, &passed_tests); run_test(led_on, &total_tests, &passed_tests); run_test(led_abc, &total_tests, &passed_tests); run_test(gpio_set, &total_tests, &passed_tests); run_test(gpio_clear, &total_tests, &passed_tests); run_test(gpio_toggle, &total_tests, &passed_tests); btn(&total_tests, &passed_tests); printf("--------------\n" "Test completed. %u out of %u tests passed.\n", passed_tests, total_tests); return CMD_RET_SUCCESS; } U_BOOT_CMD(test_nx_gpio, 1, 0, do_test_nx_gpio, "Test cmd_nx_gpio", "\nRun tests for cmd_nx_gpio\n" ", which include gpio get, led on¦off, gpio set¦clear¦toggle, and btn\n" );

Code Block 2 test/cmd_nx_gpio.c

## 5.3 Run test

$ test_nx_gpio

## 5.4 Test result

The result table is below.

Test result table

- Pass: Test case worked as "expectation" of the test scenario table

- Fail: Test case did not work as "expectation" of the test scenario table

| - Case | - Result |
| --- | --- |
| - get GPIOA3 input | - Pass |
| - led off | - Pass |
| - led on | - Pass |
| - led abc | - Pass |
| - set GPIOA4,5 | - Pass |
| - clear GPIOA4,5 | - Pass |
| - toggle GPIOA4,5 | - Pass |
| - toggle GPIOA4,5 on button press | - Pass |

LEDs  successfully turned on and off before and after each GPIO output command

# Reference

- tsb1101_control_v100_rel_20181126.pdf

- S5P6818_Datasheet_Ver142A.pdf

- https://docs.u-boot.org/en/latest/

- https://github.com/NexellCorp/u-boot-2016.01